#include <linux/module.h>

#include <linux/version.h>

#include <linux/init.h>

#include <linux/fs.h>

#include <linux/cdev.h>

#include <linux/uaccess.h>

#include <linux/gpio.h>

#include <linux/delay.h>

/* Meta Information */

MODULE_LICENSE("GPL");

MODULE_AUTHOR("Johannes 4 GNU/Linux");

MODULE_DESCRIPTION("A driver to write to a LCD text display");

/* Variables for device and device class */

static dev_t my_device_nr;

static struct class *my_class;

static struct cdev my_device;

#define DRIVER_NAME "lcd"

#define DRIVER_CLASS "MyModuleClass"

/* LCD char buffer */

static char lcd_buffer[17];

/* Pinout for LCD Display */

unsigned int gpios[] = {

3, /* Enable Pin */

2, /* Register Select Pin */

4, /* Data Pin 0*/

17, /* Data Pin 1*/

27, /* Data Pin 2*/

22, /* Data Pin 3*/

10, /* Data Pin 4*/

9, /* Data Pin 5*/

11, /* Data Pin 6*/

5, /* Data Pin 7*/

};

#define REGISTER_SELECT gpios[1]

/**

* @brief generates a pulse on the enable signal

*/

void lcd_enable(void) {

gpio_set_value(gpios[0], 1);

msleep(5);

gpio_set_value(gpios[0], 0);

}

/**

* @brief set the 8 bit data bus

* @param data: Data to set

*/

void lcd_send_byte(char data) {

int i;

for(i=0; i<8; i++)

gpio_set_value(gpios[i+2], ((data) & (1<<i)) >> i);

lcd_enable();

msleep(5);

}

/**

* @brief send a command to the LCD

*

* @param data: command to send

*/

void lcd_command(uint8_t data) {

gpio_set_value(REGISTER_SELECT, 0);	/* RS to Instruction */

lcd_send_byte(data);

}

/**

* @brief send a data to the LCD

*

* @param data: command to send

*/

void lcd_data(uint8_t data) {

gpio_set_value(REGISTER_SELECT, 1);	/* RS to data */

lcd_send_byte(data);

}

/**

* @brief Write data to buffer

*/

static ssize_t driver_write(struct file *File, const char *user_buffer, size_t count, loff_t *offs) {

int to_copy, not_copied, delta, i;

/* Get amount of data to copy */

to_copy = min(count, sizeof(lcd_buffer));

/* Copy data to user */

not_copied = copy_from_user(lcd_buffer, user_buffer, to_copy);

/* Calculate data */

delta = to_copy - not_copied;

/* Set the new data to the display */

lcd_command(0x1);

for(i=0; i<to_copy; i++)

lcd_data(lcd_buffer[i]);

return delta;

}

/**

* @brief This function is called, when the device file is opened

*/

static int driver_open(struct inode *device_file, struct file *instance) {

printk("dev_nr - open was called!\n");

return 0;

}

/**

* @brief This function is called, when the device file is opened

*/

static int driver_close(struct inode *device_file, struct file *instance) {

printk("dev_nr - close was called!\n");

return 0;

}

static struct file_operations fops = {

.owner = THIS_MODULE,

.open = driver_open,

.release = driver_close,

.write = driver_write

};

/**

* @brief This function is called, when the module is loaded into the kernel

*/

static int __init ModuleInit(void) {

int i;

char *names[] = {"ENABLE_PIN", "REGISTER_SELECT", "DATA_PIN0", "DATA_PIN1", "DATA_PIN2", "DATA_PIN3", "DATA_PIN4", "DATA_PIN5", "DATA_PIN6", "DATA_PIN7"};

printk("Hello, Kernel!\n");

/* Allocate a device nr */

if( alloc_chrdev_region(&my_device_nr, 0, 1, DRIVER_NAME) < 0) {

printk("Device Nr. could not be allocated!\n");

return -1;

}

printk("read_write - Device Nr. Major: %d, Minor: %d was registered!\n", my_device_nr >> 20, my_device_nr && 0xfffff);

/* Create device class */

if((my_class = class_create(THIS_MODULE, DRIVER_CLASS)) == NULL) {

printk("Device class can not be created!\n");

goto ClassError;

}

/* create device file */

if(device_create(my_class, NULL, my_device_nr, NULL, DRIVER_NAME) == NULL) {

printk("Can not create device file!\n");

goto FileError;

}

/* Initialize device file */

cdev_init(&my_device, &fops);

/* Regisering device to kernel */

if(cdev_add(&my_device, my_device_nr, 1) == -1) {

printk("lcd-driver - Registering of device to kernel failed!\n");

goto AddError;

}

/* Initialize GPIOs */

printk("lcd-driver - GPIO Init\n");

for(i=0; i<10; i++) {

if(gpio_request(gpios[i], names[i])) {

printk("lcd-driver - Error Init GPIO %d\n", gpios[i]);

goto GpioInitError;

}

}

printk("lcd-driver - Set GPIOs to output\n");

for(i=0; i<10; i++) {

if(gpio_direction_output(gpios[i], 0)) {

printk("lcd-driver - Error setting GPIO %d to output\n", i);

goto GpioDirectionError;

}

}

/* Init the display */

lcd_command(0x30);	/* Set the display for 8 bit data interface */

lcd_command(0xf);	/* Turn display on, turn cursor on, set cursor blinking */

lcd_command(0x1);

char text[] = "Hello World!";

for(i=0; i<sizeof(text)-1;i++)

lcd_data(text[i]);

return 0;

GpioDirectionError:

i=9;

GpioInitError:

for(;i>=0; i--)

gpio_free(gpios[i]);

AddError:

device_destroy(my_class, my_device_nr);

FileError:

class_destroy(my_class);

ClassError:

unregister_chrdev_region(my_device_nr, 1);

return -1;

}

/**

* @brief This function is called, when the module is removed from the kernel

*/

static void __exit ModuleExit(void) {

int i;

lcd_command(0x1);	/* Clear the display */

for(i=0; i<10; i++){

gpio_set_value(gpios[i], 0);

gpio_free(gpios[i]);

}

cdev_del(&my_device);

device_destroy(my_class, my_device_nr);

class_destroy(my_class);

unregister_chrdev_region(my_device_nr, 1);

printk("Goodbye, Kernel\n");

}

module_init(ModuleInit);

module_exit(ModuleExit);